An Overview of The Indonesian WiFi Regulation

Onno W. Purbo

Based On:

KEPMEN 2/2005 tentang Penggunaan Pita Frekuensi 2.400-2.483.5MHz

Ministry Act No. 2/2005 on The Use of Frequency Band 2.400-2.483.5MHz

The Principle Usage:

- For Data & Internet Access

- Sharing

- Non-interfering.

- Must Follow Technical Requirement from the Ministry.

Technical Requirement

- EIRP for Outdoor 36dBmW

- EIRP for Indoor 27dBmW

- Max TX Power 100mW (20dBm)

- Out of Band Emission Max. -20dBc/100kHz.

No need to pay any tax / license fee.

All equipment must be type approved / certified.

All existing Microwave Link has 2 years period to reassign to different frequency.

No protection from interference will be provided by the government.

Government will monitor the interference &  frequency usage.

Signed By M. Hatta Rajasa 5 January 2005.