# MULTIMEDIA TRAINING KIT

Exercises: Access Point Configuration

Developed by: Sebastian Buettrich

Imagine a Typical situation:

- There is an existing Internet connection, which could be a fixed line, a DSL, or a satellite connection.

- You want to enable many users to share this line via a wireless Access Point, which is connected to that line.

What you will need:

PC, browser, and away to connect to the Access Point (wireless or via cable)

Steps:

connect to the AP - you will need to know the default settings. Please write these down:

WAN IP:

LAN IP:

SSID:

Channel:

is WEP used?

is DHCP enabled?

Give the Access Point a New Password! Now!:)

write it down here:

Change the IP settings - make the Access Point work in the 10.11.12.x subnet!

adjust your computers settings, if needed, to reconnect to the AP.

Change the Channel so it works on Channel 10 - if that s allowed in your country.

adjust your computers settings, if needed, to reconnect.

Change the SSID to a name of your choice:

write it down here:

adjust your computers settings, if needed, to reconnect.

Use the MAC address control, so it only allows you and your neighbour on the Access Point.

write down your MAC addresses: